| COMPANY/ PROJECT NAME or ID/ LOCATION ( City, State) |  | DATE | NEW REVISED |  |  | PAGE 1 of 2 |
| --- | --- | --- | --- | --- | --- | --- |
| WORK ACTIVITY (Description): FALL PROTECTION |  |  |  |  |  |  |
| DEVELOPMENT TEAM | POSITION / TITLE | REVIEWED BY: |  | POSITION / TITLE |  |  |
| MINIMUM REQUIRED PERSONAL PROTECTIVE EQUIPMENT |  |  |  |  |  |  |
| REFLECTIVE VEST HARD HAT LIFELINE / HARNESS SAFETY GLASSES | GOGGLES FACE SHIELD HEARING PROTECTION SAFETY SHOES | AIR PURIFYING RESPIRATOR SUPPLIED RESPIRATOR PPE CLOTHING |  |  | GLOVES Cut-resistant OTHER |  |
| ¹JOB STEPS | ²POTENTIAL HAZARDS | ³CRITICAL ACTIONS TO MITIGATE HAZARDS |  |  |  |  |
| - Access task / job |  | - Plan the task, identifying the work area and ensure it is clear of any obvious slip and trip hazards. - The person required to work at height must complete a task based risk assessment prior to commencing work. The risk assessment must consider all alternative fall prevention hazard controls before electing to use a fall arrest system. The use of a fall arrest system should be the last preferred hazard control. - Fall arrest systems must NOT be used unless there are more than 2 people on site at all times when using the system. |  |  |  |  |
| - Inspect fall arrest equipment |  | - A thorough inspection of all equipment must be conducted prior to its use. Inspections should check for; - General wear and tear - Cuts and tears to webbing - Signs of damage to stitching - Stretching of the lanyard. - Welding burns to webbing |  |  |  |  |
| - Identify / Install an anchor point |  | - Person responsible to identify / install an anchor point must do so without risk of falling. Elevated work platforms, ladders or scaffold must be used to access the anchor point. - Only approved anchor points and slings can be used as an anchor point or attachment to the anchor point. - The anchorage point of the fall arrest system should be positioned to ensure that the static restraint line does not allow the person wearing the system to free-fall. - Fall prevention to be used when working under 6.4M, rather than only fall arrest. |  |  |  |  |
| - Install static line |  | - Only approved static line material (rope, webbing, cable), attachments and tensioners can be used for a fall arrest static line. - Access to the static restraint line of the system must be provided without the risk of falling. |  |  |  |  |
| - Fit full body harness |  | - The full body harness must be fitted as per the manufacturers instructions. |  |  |  |  |
| - Fit full body harness |  | - The full body harness must be fitted as per the manufacturers instructions. |  |  |  |  |
| - Rescue of a fallen worker |  | - Access equipment must be available to access a fallen worker on site at all times when using a fall arrest system. - Adequate means of access include, elevated work platforms, forklift with approved man cage, and crane with man box or mobile scaffold. - Adequately trained workers must also be available to operate any access / rescue equipment. |  |  |  |  |